# Assignment 01

# Site Modeling with LiDAR and Physical Fabrication

Course: Arch 565 - Advanced Computer Applications II
Due Date: Tuesday, February 17th
Weight: 25% of Assignments grade

## Overview

This assignment challenges you to combine emerging digital documentation technologies with traditional physical modeling techniques to create a hybrid site model. Using LiDAR scanning, you will capture a portion of the built environment and its topography, process the data into a clean 3D model, and translate it into a physical representation through digital fabrication methods. The goal is to develop skills in spatial accuracy, digital-to-physical workflows, and critical reflection on the role of computational technologies in architectural practice.

## Objectives

- Gain hands-on experience with LiDAR-based site documentation.

- Process point cloud data into usable 3D models for architectural analysis.

- Integrate digital modeling and computational fabrication to produce a physical site model.

- Reflect on the accuracy, limitations, and opportunities of LiDAR technology compared to traditional site modeling.

https://spacesymmetrystructure.wordpress.com/rheotomic-surfaces/

## Assignment Tasks

- Site Selection & Scanning

- Gather LiDAR data for your project site through publicly available sources.

- OR:

  - Select a small urban or landscape site (approx. 50’ x 50’ minimum).

  - Use a LiDAR-enabled device (iPad Pro, LiDAR scanner, or equivalent) to capture the site, including topography, vegetation, and built features.

  - Perform multiple scans to ensure complete coverage and minimize data gaps.

- Data Processing & Digital Modeling

- Import the LiDAR data into appropriate software (e.g., Autodesk Recap, CloudCompare, Rhino, Grasshopper).

- Clean and align the point cloud, removing noise and filling gaps.

- Generate a simplified but accurate 3D model suitable for fabrication (surface mesh or contour model).

- Physical Fabrication

- Translate the digital model into fabrication-ready files.

- Use laser cutting, CNC milling, or 3D printing to produce a physical site model at a scale of your choosing

- Assemble and finish the physical model, ensuring clarity and legibility of site features.

- Documentation & Reflection

- Submit a short (2-3 page) illustrated report or journal documenting your workflow, including scanning process, data cleaning, model preparation, fabrication, and assembly.

- Critically evaluate the strengths and limitations of LiDAR technology in architectural site analysis and how it compares to traditional surveying and hand-built modeling

## Deliverables

- Digital Submission: Cleaned point cloud, processed 3D model, and fabrication-ready files (.3dm, .dwg, or .stl) these are for your record and do not need to be uploaded.

- Physical Submission: physical site model assembled and finished.

- Report Submission: PDF (2-3 pages) with workflow documentation, images, and critical reflection

## Evaluation Criteria

- Technical Proficiency (30%) - Accuracy and clarity of LiDAR scanning, data processing, and digital modeling.

- Craft & Fabrication (30%) - Precision, scale accuracy, and finish of the physical model.

- Workflow Integration (20%) - Clear translation between digital and physical methods.

- Critical Reflection (20%) - Depth of analysis in the written report regarding opportunities and limitations of LiDAR in architectural workflows.

## Grading Rubric (100 points total)

| Criteria | Excellent (A: 90-100) | Satisfactory (B-C: 70-89) | Needs Improvement (D-F: <70) |
| --- | --- | --- | --- |
| Technical Proficiency (30 pts) | Point cloud capture is complete, accurate, and well-processed; 3D model is clean, detailed, and fabrication ready. | LiDAR data is captured with minor gaps; 3D model is serviceable but contains simplifications or minor errors. | LiDAR scan is incomplete or poorly processed; 3D model is inaccurate, messy, or unusable for fabrication. |
| Craft & Fabrication (30 pts) | Physical model is precise, clean, and well assembled; scale and features are legible; fabrication techniques are skillfully applied. | Physical model shows reasonable accuracy but has noticeable imperfections or assembly flaws; scale legibility is adequate. | Model is poorly fabricated, incomplete, or unclear; features are illegible; craftsmanship is lacking. |
| Workflow Integration (40 pts) | Seamless translation from LiDAR to 3D model to physical fabrication; workflow demonstrates clear understanding of digital / physical integration. | Workflow is mostly clear but has gaps; some inefficiencies or issues in moving between digital and physical steps. | Workflow is fragmented, incomplete, or shows little understanding of how digital and physical processes connect. |
| Critical Reflection (30 pts) | Report clearly documents workflow with images; reflection demonstrates deep, critical insight into opportunities and limitations of LiDAR vs. traditional methods. | Report documents workflow but with limited detail; reflection is descriptive rather than analytical, with surface level critique. | Report is incomplete, unclear, or missing; reflection lacks insight, depth, or meaningful evaluation. |